# Test Definitions Document:

# GFT  G3 control core (GCore)

# Based on the P650 processor

Prepared by:

S. Safonov, Y. Theodor

ChatGPT OUTSIDE

| - | First draft | 13-Jan-26 |
| --- | --- | --- |

Table 1: Table of revisions

This is an SRR level tests definition document. It is meant the global methodology. The tests will be elaborated following hoped pattern approval at the SRR, towards PDR

Contents

1	Introduction	4

1.1	Scope and overview:	4

1.2	Glossary	4

1.3	Referenced documents	4

2	Test descriptions.	4

2.1	[T001] Fast report UART	4

2.2	[T1002] Safety & security module	5

2.2.1	T1002.01: Watchdog integrity	5

2.2.2	T1002.01: Wakeup	5

2.2.3	T1002.02: Survival	5

2.2.4	T1002.02: Operation	5

2.3	T1010: FSI enslavement	6

2.4	T1020: Boot Flow	6

2.4.1	Algorithm	6

2.4.2	Action	7

2.4.3	UART Auto baud rate	7

2.4.4	CAN Auto baud rate	7

2.4.5	EtherCAT state machine	8

2.4.6	Communication Exclusivity	8

2.5	T1030: Object dictionary	8

2.6	T1040: EtherCAT link	8

2.7	T1060: Client-server interconnect	9

2.7.1	Data Interconnect Client to Server	9

2.7.2	Data Interconnect Server to Client	9

2.8	T1070 Core Synchronization	10

2.8.1	T1070.1 Synchronization	10

2.8.2	T1070.2 Enslavement to FSI	10

2.8.3	T1070.3 Enslavement to EtherCAT	11

2.8.4	T1070.4 CLA synchronization	11

2.9	T1080: Emergency and Safety	11

2.9.1	T1080.1 STO	11

2.10	T1090: Simulation ability	11

# Introduction

## Scope and overview:

This document outlines tests next-generation P650 core for GFT-Servo, referred to as “G Core.”

This document lists tests. This does not mean that each test should be performed once.

Some tests should be repeated many times, others must endure long time for stability assurances.

Mind that this document only describes tests; the STP (Software Test Plan) document describes how and when these tests are used; and the compliance table traces the tests to the specified requirements.

The tests are partitioned as:

- Unit tests, to be performed once on development, or repeated following modification of relevant SW blocks. These tests may require the presence of electronics staff using scopes etc. and may require the use of JTAG debugger.

- Release tests, to be performed on each release. These tests do not involve the opening of product cases and normally do not require special calibrated lab equipment. Tests may still require connecting cables, PS and motor to the external connections of the drive.

This document relies on: - G-Core definitions document - G-Core requirements (Excel)

For overview, refer to the definitions document

## Glossary

Refer to the spec document

## Referenced documents

| #1 | GFT G3 control core (GCore) Test descriptions | GFT |
| --- | --- | --- |

## Terminology

| Verify xxxx failure | Verify failure code xxxx is registered in the errors log; and that if motion was on, it was aborted. |
| --- | --- |

# Test descriptions.

## [T001] Fast report UART

Select time variable and interrupt counter for the Periodic report.

Set the UART rate to 3Mb/sec

Read the transmission frame rate from the EUT

Record the periodic report for 1 sec, verify time steps have the fixed and expected rate of periodic report

## [T1002] Safety & security module

### T1002.01: Watchdog integrity

| Watchdog action | Release | Block the main SW module intentionally. Verify device resets |
| --- | --- | --- |
| No external watchdog kicking | Release | Stop the Watchdog kicks at the safety module intentionally. Verify device resets |
| Protect both interrupts and BG | Release | Shut the operational interrupt process. Verify device resets. |
|  | Release | Hang the operational idle process. Verify device resets |
|  | Stop CPU#2 | When configured for debug: Nothing Otherwise: Device resets |

### T1002.01: Wakeup

| Wakeup | Unit Test | Hold STO high t external pins Verify STO remains low till control passes to operational SW |
| --- | --- | --- |

### T1002.02: Survival

| Erasure | Release | Verify a firmware download does not modify flashed configuration, e.g., limits for position and speed |
| --- | --- | --- |

### T1002.02: Operation

| Speed limit | Release | In simulation mode, Set to SW higher reference or speed limits than flashed to safety. Verify database failure |
| --- | --- | --- |
|  | Release | Program diverging control. Verify condition is caught and STO asserted |
| Position limit | Release | In simulation mode, Set to SW higher reference or position limits than flashed to safety. Verify database failure |
|  | Release | Program diverging control. Verify condition is caught and STO asserted. Do that both with position control and speed control closure modes |
| Stop deceleration | Release | In simulation mode, Set to SW higher deceleration or stop deceleration limits than flashed to safety. Verify database failure |

## T1010: FSI enslavement

| Synchronism | Unit test | Connect to a dual FSI master. Setup sync to external master. Verify by scope the timing of Rx and Tx relative to the timing pulse |
| --- | --- | --- |
|  | Release | Connect to a dual FSI master. Setup sync to external master. Check by recorder that: FSI error counter is zero over the recorder period, FSI message count is as expected, i.e. total time divided by FSI message rate, and FSI sync is always within 1usec of its expected position in the PWM frame |
|  | Release | Verify the interrupt counters at the CLA and at core 2. Verify by recorded interrupt position that it never deviates >1usec from expected position in PWM frame |
| Protocol | Release | Verify by recorder that the transmitted/received fields are similar. For the test, fix the message contents so that the received contents will be predictable. |
| Default | Release | DO NOT Setup sync to external master. Verify that FSI message counter does not update. |

## T1020: Boot Flow

### Algorithm

T1020.1

| Algorithm | Unit Test | Follow the boot sequence manually when FW is properly loaded, in the debugger. Test conformity to spec. until jump to operational SW |
| --- | --- | --- |
|  | Unit Test | Follow the boot sequence manually when FW is corrupted, in the debugger. Test conformity to spec. until selection of boot mode. |

### Action

| Action | Release | Do a full firmware download process, when FW is initially loaded. should succeed in the sense that next wakeup FW responds to operational SW normally. |
| --- | --- | --- |
|  | Release | Disrupt intentionally the CRC. Verify that the next wake remains in the boot state. |
|  | Release | Disrupt download FW intentionally by downing PS. Verify the next wake is to the boot state. |
|  | Release | Do a full firmware download process, when FW is initially at the boot. should succeed in the sense that next wakeup FW responds to operational SW normally. |

T1020.2: Do all the above tests for CAN (using PC loader)

T1020.3: Do all the above tests for UART (using PC loader)

T1020.4: Do all the above tests for EtherCAT (simple FOE loading of file from TWINCAT)

### UART Auto baud rate

T1020.5

| UART Baud rate | Release | Send char=7 with bauds 19200 to 921600. Verify correct response at the correct boot baud rate of default (230400) and programmed to 460800. |
| --- | --- | --- |

### CAN Auto baud rate

T1020.6

| UART Baud rate | Release | Send ID SDO at baud rates 250000 to 1000000. Verify correct response at the default boot baud rate of 500000, and acceptance of ID. Repeat when the baud rate is programmed to 1M |
| --- | --- | --- |

### EtherCAT state machine

T1020.7

Verify that at the boot state, the ESC will refuse to go PRE-OP and whould only approve going to the boot-loading state.

### Communication Exclusivity

T1020.8

| UART Start | Unit test | Make communication by UART. Verify that immediately after establishing UART communications CAN and EtherCAT are blocked |
| --- | --- | --- |
| CAN Start | Unit Test | Make communication by CAN. Verify that immediately after establishing CAN communications UART and EtherCAT are blocked |
| EtherCAT start | Code review | Make sure that after boot transits from the Init EtherCAT state, UART and CAN are blocked |

## T1030: Object dictionary

| Object access | Release | For each of the list of supported objects, set it: - To values out of the supported min max. Should be rejected - To at least 2 values within range. Acceptance should be verified by interrogating core#1 through memory reads, with addresses taken from map file or from elf. |
| --- | --- | --- |

## T1040: EtherCAT link

TBD

|  | Release |  |
| --- | --- | --- |

## T1060: Client-server interconnect

### Data Interconnect Client to Server

#### T1060.1 Virtual Sensor slots

| Virtual sensor slots | Release | Test that values put into the slot can be configured as sensor outputs by core #1 |
| --- | --- | --- |

#### T1060.2 SEAL standard interface

| SEAL standard interface | Release | Test that every value in the command structure is correctly accepted by core #1. Test interfaces for both axes. Use .map or elf for address of data to read. |
| --- | --- | --- |

#### T1060.3 Objects interface stack

| Objects interface stack | Unit Test | Go through the code and verify the command – response stack usage |
| --- | --- | --- |
|  | Release | Put through the interpreter commands that set variables to Core#1. Readback (through memory service) the correct variables from Core#1 |
|  |  | Intentionally overflow the stack. Verify STACK_FULL error. |

#### T1060.4 Client inventory

| Client inventory | Unit Test | Test that values put by the client correctly reflect the expected allocation to CPU#2 |
| --- | --- | --- |

### Data Interconnect Server to Client

#### T1060.5 SEAL standard interface

| SEAL standard interface | Release | Test that every value in the feedback / setup report is correctly accepted by core #1. Test interfaces for both axes. Use .map or elf for address of data to set in Core#1. |
| --- | --- | --- |

#### T1060.6 SEAL Recorder interface

| Recorder buffer | Release | Program SigType and memory records into the interface. Operate recorder and correlate direct memory reads (by map file) with recorder contents. |
| --- | --- | --- |

#### T1060.7 ADC raw data

| ADC raw data | Release | correlate direct memory reads (by map file) with interface contents. |
| --- | --- | --- |

#### T1060.8 Server inventory

| Server inventory | Unit Test | Test that values put by the server correctly reflect the expected values of CPU#2 |
| --- | --- | --- |

#### T1060.9 Data Integrity Mechanisms

| Data Integrity mechanism | Unit Test | Step through the code and verify that IPC buffers are only treated as whole. Verify flag setting on buffer read, and flag logic adherence in buffer reads. |
| --- | --- | --- |

## T1070 Core Synchronization

### T1070.1 Synchronization

| Core synchronization | Release | Record the IPC counter as sampled by the ISR at both core simultaneously, tagged by the axis in effect. Verify (1) interleaving: core #2 services axis#2 while Core#1 services Axis#1 and vice versa. (2) This is correct even if axis#2 is absent, where the axis#2 ISR does only minimal job. (3) timing difference between consecutive interrupt instances is the configured sampling time +/-5usec, and +/- 0.01usec on average. (4) timing match to 5usec between core readings. |
| --- | --- | --- |

### T1070.2 Enslavement to FSI

| Enslavement timing | Release | Repeat T1070.1 with FSI enslavement. Match to the FSI master time was already tested in T1010 |
| --- | --- | --- |

### T1070.3 Enslavement to EtherCAT

| Enslavement timing | Release | Repeat T1070.1 with EtherCAT enslavement while in DC. In addition, Match the ECAP capture of SYNC0 to the ECAP catch on ISR. Time should not be on average > 5usec and never > 20usec. |
| --- | --- | --- |

### T1070.4 CLA synchronization

| CLA timing | Release | Synchronize to FSI, introduce intentional 5usec jitter into FSI sync pulse. Read PWM counter of Core#1 together with sync PWM for CLA and verify value close < 1usec. |
| --- | --- | --- |

## T1080: Emergency and Safety

### T1080.1 STO

| STO timing | Release | Verify that after STO start is detected, STO activation occurs after nominal grace time +/-100usec. |
| --- | --- | --- |
| STO action | Release | Set current to motor (torque mode) De-assert STO deliberately by CPU#2 and wait grace time. Verify that the motor is shorted by observing its decaying phase currents on recording |

## T1090: Simulation ability

| Simulation | Release | Use SolFlow to configure virtual motor and sensor. Go through the current tuning process, than through commutation, than tune speed loop. Observe speed step similar to the Simulink result for the same model. Match should be to 0.5% of the full speed command. |
| --- | --- | --- |

# TBD table

The following issues, for later implementation, do not yet have their tests defined:

- EtherCAT

- Compilation in Visual environment for use as virtual drive (simulation mode)