# Initial thoughts:

# GFT  G3 control core (GCore)

# Based on the P650 processor

Prepared by:

S. Safonov, Y. Theodor

ChatGPT OUTSIDE

| - | First draft | 13-Jan-26 |
| --- | --- | --- |
| 1.0 | Elaborate FSI enslavement mode | 14-Jan-26 |
| 1.1 | JH comments | 14 Jan 2026 |
| 1.2 | Add multiple axis by Ethercat and HW requirements | 14-Jan-26 |
| 1.3 | Update boot sequence after discussion w. Shmuel | 14-Jan-26 |

Table 1: Table of revisions

Contents

1	Introduction	3

1.1	Scope:	3

1.2	Glossary	3

2	Functional description	4

2.1	Core roles	4

2.1.1	Cores	4

2.1.2	Core interconnects	5

2.2	Motion controller (CPU1 + CLA or HOT) description	6

2.2.1	General features:	6

2.2.2	Safety security module	7

2.2.3	FSI enslavement	7

2.2.4	Boot flow	9

2.2.5	Firmware downloading	9

2.3	The Client (CPU 2)	10

2.3.1	Client roles	10

2.3.2	Multiple axes and EtherCAT	10

2.4	Client-server interconnect	11

2.4.1	Data interconnect	11

2.4.2	Core synchronization	12

3	Appendix: HW requirements	13

4	Appendix: Roadmap and Work to do	14

4.1	Mapping of DftServo feature for reuse	14

4.2	Feature updates	14

4.3	Feature families	16

# Introduction

## Scope:

This document outlines the next-generation P650 core for GFT-Servo, referred to as “G Core.”
While based on the existing GFT-Servo architecture, it introduces a new design direction.

- The P650 revision focuses on several key objectives:

- Establishing a unified control core across the entire GFT product family

- Providing limited built-in safety features, with certified safety functionality delegated to an external high-level CPU

- Standardizing low-level communication interfaces

- Enabling a wide range of applications through a flexible, dynamically configurable application core

This document is an early draft intended to confirm alignment on the overall direction before we invest in the extensive, time-consuming detailed work. It reflects the first day’s results of formal architecture discussions, consolidating and structuring the many informal conversations of recent weeks. Now it also contains corrections by colleague responses. It does not aim to be highly precise, comprehensive, or fully refined.

## Glossary

| Term | Means |
| --- | --- |
| BIT | Built In Test |
| CAN | Control Area Network |
| CAN FD | CAN with Frequency with Flexible Data rate |
| CLA | Control Law Accelerator, a dedicated coprocessor for algorithm number crunching |
| DS402 (now IEC61800-7) | A general device profile (ICD) for a servo drive |
| EEPROM | Electronic-Erasable Programmable Read Only Memory |
| EtherCat | A determistic high speed serial communication that may run on Ethernet hardware. |
| FMMU | Field Memory Management Interface An EtherCat unit the directly controls the transfers between message partitions to specific CPU-accessible memory location |
| G3 or GCore | Generation 3 G1 was before GFT servo SW G2 is the present GFT servo SW |
| HOT CPU | On systems which require electrical isolation, a separate CPU that replaces the CLA functionality beyond the insulation barrier |
| HW | Hardware |
| ICD | Interface Control Document, details communication formats and contents. |
| I2C | Inter Integrated Circuit, a primitive open collector serial communication |
| IPC | Inter Processor Communication: a dual ported enhanced message box between the two cores of the P650 |
| MAC | Media Access Controller. An on board peripheral that translates messages at the CPU side to physical media requirements |
| PHI | "Physical" – by the commands of a MAC, produce actual signals on the physical transmission lines. |
| P650 | A core by TI, intended for next gen use |
| RMII | Redudec Medeia Independent Interface, a protocol serving between MAC and PHI |
| SW | Software |

# Functional description

## Core roles

### Cores

The 650 SW is made of 3 cores:

Core#1:  Speed and Position:

- Control functions,

- sensor interfaces, (Hall , Encoders, IMU)

- reference profilers (single or dual axes)

- safety part, (optional)

CLA: Current control (single or dual axes)

Or:

Hot CPU: Current control (single or dual axes)

Core#2: Services:

- Communications, protocols,

- interpreters,

- user programs VM,

- external interfaces

### Core interconnects

Core#1 to CLA interconnect: Via CLA mailboxes and shared memory

Or

Core#1 to HOT CPU interconnect: Via UART, 3 to 6MB/sec

Core#1 to Core#2 interconnect: Through IPC

Figure 1: Core arrangement. HOT CPU is dashed; it is a replacement to CLA work, on applications where electrical isolation is required.

Notes:

- The USB connection, although planned, is not shown. From the controller’s perspective, USB functions simply as a high-speed UART interface.

- When two axes operate on the same core, they share the same sampling period and remain synchronized. Each axis receives its own interrupt, with the interrupts staggered by half a cycle.

- The HOT CPU interface is currently defined only at the hardware-allocation level. There is no near-term requirement for HOT CPU functionality, so detailed development is postponed. A HOT CPU does not benefit from the tight coupling available to the CLA; it intentionally trades control bandwidth for galvanic isolation. Because a typical message to the HOT CPU is around 16 bytes, transmission requires roughly 60 µs, resulting in a total delay of up to 100 µs. Consequently, systems using a HOT CPU will have their velocity-loop bandwidth limited to approximately 500 Hz, which is acceptable for most applications.

## Motion controller (CPU1 + CLA or HOT) description

### General features:

The controller core is designed with the following principles:

• 	Structural isolation: The control architecture is kept independent from project‑specific logic to simplify maintenance.

• 	Strict command model: The core accepts commands exclusively through an object dictionary and performs no high‑level command interpretation.

• 	Standardized machine interface: The core is intended solely for machine‑level interaction, using a standardized interface (object dictionary, command structure, feedback structure, and setup‑report structure) compatible with SEAL implementations.

• 	Partial safety integration: Although not intended for full safety certification, a safety module is embedded within the compact direct‑control code and operates as an independent module. It has its own periodic service (via CpuClock2) and a software watchdog.

Low‑grade safety compliance: Due to its focused scope, the control core can be fully specified for lower‑tier safety requirements. High‑level standards such as ISO 26262 or IEC 61508 ASIL4 are not feasible here and would require a C29xx‑class architecture.

- The control core does not perform current control which is handled either by the CLA or by a “HOT” CPU via communication.

### Safety security module

The safety module:

- Operates as an independent process with its own dedicated flash sector.

- Uses minimal code and runtime resources.

- Is not replaced during standard software updates.

- Reserves the watchdog and CPUTimer2 exclusively for its own operation, preventing conflicts and ensuring it can respond even if the main application blocks interrupts or starves it of CPU time.

- Can be configured through parameters such as maximum speed, position limits, stop deceleration, encoder scaling, and matching information.

### FSI enslavement

The FSI real time command channel allows a remote master to directly control core#1 without core#2 mediation. This is done where the main role of core#2, i.e. interface and process control, is taken by a remote master, and passing through core#2 will:

- Add unnecessary delays

- Burden the core#2 SW, which should be cheap to maintain, a strictly timed process.

For this purpose, the FSI communication works as strict reference generator, in one of 
IEC61800-7  CP(Cyclic Periodic) modes.

The FSI is one-lane wide, and bidirectional. Its back path carries feedback and status, namely position, speed, current, voltage, power consumption and status bits, as well as slots for programable reports. A part of the FSI communication (always 15 words) is devoted to object dictionary set/get, only for 2-words (long or float) objects. All the objects concerned with CP motion are accessible via 2-word objects.

The FSI timing must be expanded to 2-axis use:

Figure 2: FSI connection and timing

Notes:

- Reliable FSI is better with a master timing pulse as in the diagram above

- The use of a master timing pulse is not a must. Timing can be locked, as in Moshe (no timing pulse there) on the communication packet itself

- At this mode, the slave sampling time adjusts and synchronizes to the FSI communication (a clock shift up to 300ppm, greater deviation creates an exception). As core#2 is timed by core#1, the core#2 timing is also automatically synchronized to the master's timing

### Boot flow

The diagram for CPU#1 is below:

Figure 3: Boot flow diagram

Notes:

- CPU2 does not have a boot. The operational SW will make its initialization, then waits an infinite loop for CPU1 to announce initialization complete.

- CPU2 does not have independent flash and PLL settings

- If CPU1 remains in FW download mode, CPU2 is in reset

### Firmware downloading

CPU1 is the master CPU.

CPU1 normally owns no communications (but the HOT CPU option and FSI slave port). In boot mode, it needs to communicate via EtherCAT or CAN. This means that the basic EtherCat and CAN handlers need be programmed into CPU1 boot – it needs be larger than usual.

CPU1 is the only one that performs FW update.

In boot mode it allocates to itself the entire set of flash blocks, and programs the FW for CPU2 as well. For this reason, CPU2 has no FW download mode.

The responsibility of matching versions between the cores is entirely the PC-side loader program. Although the FW downloading is dumb and it is possible to program any CPU alone, no mechanism exists in the embedded device to enforce CPU version matching. It is advisable to update both CPU always, and to defer the writing of both their statistics modules to be last.

Note:

The statistics block is erased first and re-written last.

This way, on partial loads, e.g., by on-loading power failure, the SW integrity check by the boot SW will fail, and the faulty load module will never run; the device will remain in boot mode, ready for FW downloading retry.

FW update option should be supported by all “config” communication protocols  via core 2

## The Client (CPU 2)

### Client roles

The client will run

- Clocking and initialization of any peripheral CPU2 uses

- All operational communication interfaces

- Interpreters that map communicated objects to server and local objects

- Any state machines other than the generic drive DS402

- Any user programs (microPython, SEAL, Legacy)

- Nonstandard sensor interface (resolver, potentiometers, etc)

- IMU interface, inertial integration TBD

- Target filtering or any other operational path calculations

- Any mission specific BIT

### Multiple axes and EtherCAT

Each CPU may run 2 servo axes.

Each CPU has one EtherCat, with 8 x FMMU and 8 x Sync Managers.

This makes sense as each axis has two Rx PDO, to Tx PDO, and 4 sync managers for service objects and bulk data.

When going to larger systems, say 4 axes, one EtherCat interface would not suffice, so each axis pair will run its own EtherCat.

A possible solution for a 4-axis system is depicted below:

Figure 4: Four axes EtherCat system

Note that the two CPUs are directly connected by RMII lines.

## Client-server interconnect

### Data interconnect

IPC interconnect will consist of the following:

Client (CPU2) to server (CPU1):

- Dual Object interfaces. Objects use the IPC set/clear/ack mechanism.

- Dual virtual sensor slots. If a non-standard (such as resolver) is used, it will be decoded by CPU2 as application

- Command interface, SEAL compatible

Server (CPU1)  to client (CPU2)

- Dual feedback & status interfaces, SEAL compatible

- Dual configuration report buffers, SEAL compatible. These buffers reflect commonly used the control limitations: current limits, position limits, speed, acceleration and jerk limits etc.

- Recorder buffer variables (32 slots). Recorder data shall be placed there by one or two axes, the same buffer.

- Raw ADC data as shorts (if client consumes ADC, it needs know the scaling)

- Server inventory

  - Number of axes

  - Sensor configuration for each axis

- Client inventory:

  - List of peripherals and IO allocated for the server

  - List of memory banks allocated to the server

  - PLL setting

Data integrity

Each non-atomic data unit (e.g., feedback buffer report) has an update 16bit counter. The counter increments and becomes odd when the buffer starts update, and is again incremented to even value on update completion. This way any data consumer must verify that the counter is even before data fetch; and that the counter value remained until after fetch completion.

The counters provide also a mean to detect overflow (counter incremented more than 2 between consecutive reads). Overflow detection and response strategies for Core#2 are left for the application designer.

If Core #1 detects update fault while a motor is on (client counters do not update as expected), it should throw an exception by the predefined DS-402 abort mode. A timing event in one axis will throw both axes.

Timing exception is legal when all motors are off – it may happen legally (e.g., on debugging)

### Core synchronization

#### General

The server core times it all. Core#2 is not allowed to affect Core#1 control timing.

The server core will program a spare PWM to issue interrupts to the clients.

There will be two interrupts:

- Axis #1 interrupt, periodic by TS

- Axis #2 interrupt (optional): periodic by TS, offset by TS/2 to 1st interrupt

The client will be required to arrange its real-time in synchronism to the server.

A client cycle is interleaved with the server cycle:

When the client runs axis#1 code the server runs axis#2 and vice versa, this way each side enjoys fresh data from the other.

Each axis shall apply the recorder separately; and increment the buffer pointer as needed. Start/Trigger/Ending conditions will be checked and applied by axis #1 only.

#### Adjustment by enslavement

Core #1 may be synchronized by external events, adapting the sampling time up to 300ppm. This may be either by FSI enslavement or by EtherCat enslavement.

For FSI enslavement, it can only happen between GFT devices, so we can mandate jitter-free transmissions and compatible sampling times.

As per EtherCat, sampling time adjustment (again up to 300ppm) will be only under distributed clock conditions, and if the period time of the EtherCat is an integer multiple of the controller sampling time

#### CLA synchronization

The CLA runs current loop. The sampling time of core#1 refers speed and position loops.

The CLA is synchronized to the slower rate of the CPU. It can run, however, at any period that is an integer division of the CPU control period.

Angle feedback to the CLA will only update at the CPU rate.

# Appendix: HW requirements

The following hardware peripherals are required to support the complete set of G-Core features, with 2-axes per CPU. Partial implementations may omit features not needed for a particular customization.

| Peripheral | Instance | Per axis | Per unit |
| --- | --- | --- | --- |
| EtherCat (1) | EtherCat |  | 1 |
| CAN (2) | Operational |  | 1 |
|  | Debug |  | 1 |
| UART(4) | IMU |  | 1 |
|  | Hot CPU | 1 |  |
|  | UI (USB) |  | 1 |
| GPIO/PWM (12) | UART DIR Control |  | 4 |
|  | STO |  | 1 |
|  | FSI Sync |  | 1 |
|  | Limit Switch | 2 |  |
|  | Shunt control | 1 |  |
| FSI (2, full set) | RX, 1 lane, slave interface |  | 1 |
|  | TX, 1 lane, slave interface |  | 1 |
|  | RX, 1-lane, extensions |  |  |
|  | TX, 1-lane, extensions |  |  |
| Encoder interface (EQEP) (2) Full set: A/B/Z |  | 1 |  |
| SPI (4) | COPI only: BISS interface (complemented by CLB) | 1 |  |
|  | Serial RAM (e.g. IS66WVS4M8BLL-104NLI-TR) | 1 |  |
|  | Serial flash for parameters | 1 |  |
| I2C (1) | EEPROM for parameters | 1 |  |

Table 2: Communication and sensor interfaces

In addition:

- PWMs for transistor drive 2x3 per axis

- SDFM x 3 per axis, each having a PWM as SDFM clock

- Analog input for DC voltage, input current, motor phase currents, moto phase voltages, temperatures, HW version indication, …

- DAC outputs as required for phase voltage indications.

# Appendix: Roadmap and Work to do

## Mapping of DftServo feature for reuse

| Projects | Features | remarks |
| --- | --- | --- |
| DOL | Ethercat with Ds402 | Ethercat supports single axis only |
| GPT | Latest position control and basic sensor-less, MCAN drivers |  |
| Dugit | Reading Biss by SPI+CLB | Not clear |
| Kevesh | Listener of Biss by CLB | Latest CLB version |
| PIT202x | Latest User program implementation, DFT_Compiler |  |
| DftTuner | DFT_IDE: sources of User program compiler and IDE |  |
| Moshe | FSI enslaving |  |

## Feature updates

Immediate (1): required for first P650 release, to operate GPT650

Immediate (2): required to operate GPT650, customer distribution

Optimization: deferred to later stage

Removal: explicitly deprecated, no migration planned

| Feature | Change | Priority |
| --- | --- | --- |
| Division to cores | Partition SW as specified | Immediate (1) |
| CPU1 dictionary | Add separate dictionary for commissioning variables | Immediate (1) |
| Standard Clarke-Park transformations, correct I/Q definitions | From non-standard implementation | Immediate (1) |
| PWM dead time protections | New | Immediate (1) |
| CAN | Set to MCAN with FD option | Immediate (1) |
| Configuration sector for operational SW | Define memory block allocations, Is burned in CCS IsHotCpu Assumed HW peripherals Peripherals allocated to CPU2 SW start address SW end address SW revision PLL settings Product code IsDigitallySigned, digital signature Adler32 checksum of SW Adler32 checksum of configuration | Immediate (1) |
| Device data | We will be supported with the new enhancements. Shall be given a specialized sector inside the device flash | Immediate (1) |
| Interrupt timing | Changed interrupt scheme | Immediate (1) |
| Control code untangle pre-emptions | Control interrupt is to be non-preempted. Inter interrupt preemption and thread safety controls to be removed | Immediate (1) |
| Flash drivers | Revised | Immediate (1) |
| Recorder | External interface exists in Core #2. Core #1 places recorder results in IPC, to be used by CORE#2. This way one recorder session may present data from both cores in synchronism. Recorder list length increased to 32 | Immediate (1) |
| CAN driver to MCAN | Revised drivers | Immediate (1) |
| Dictionary support of multiple axes | According to DS402, other axis has similar dictionary with address offset | Immediate (2) |
| Exception handling | Sensor and other exceptions shall apply even if the motor is off. An exception on motor off shall be timed out; If a failure disappears for a given time, it is deleted. Motor start shall not be attempted under failure conditions. | Immediate (2) |
| Sensor arbitration between peripherals | Encoder sockets shall be shared between the axes; there is no predefined socket-axis association. It will be checked that a socket is not used twice. | Immediate (2) |
| Boot | Flow diagram revised | Immediate (2) |
|  | FW downloading revised | Immediate (2) |
| EtherCat | Direct interface, not through IPC | Immediate (2) |
| Battle short | Relaxation of driver-preserving limits such as over-temperature by mode | Immediate (2) |
| FSI | Expansion to 2-axis timing | Immediate (2) |
| Serial storage RAM on SPI | For recorded data | Optimization |
| CAN FD DS1301 | Update to CAN FD standards (SDO and block logics revised, PDO enlarged…) | Optimization |
| Position profiler | From int64 to double | Optimization |
| Assembly code | Removal | Optimization |
| High speed DMA based process data transmission | Addition | USB support: Immediate Ethercat support: Optimization |
| RT FFT analysis for identification | Removal, to be replaced by PC code | Optimization |
| Shunt management | Each axis reports its predicted power requirement, coordinated shunt or burn-over-the-motor management | Optimization |

## Feature families

Sensors

| Sensor | Features | Remarks |
| --- | --- | --- |
| Quad encoder | 2 encoders |  |
| Halls sensors |  | Fix implementation as universal sensor |
| Resolver |  | No recorded request at recent years. Supported by legacy systems like SK To be removed |
| Potentiometer |  | No recorded request at recent years Supported by legacy systems like SK To be removed |
| Tachmeter | No support planned |  |
| Abs encoder | SSI,BISS 2 encoders (mix with Quad encoder) | To implement reading by CLB only, introduce full parametrization |
|  | ENDAT | No support planned |
|  | TAMAGAWA | No support planned |
| Analog halls (sin/cos) | Implementation exists, never tested in practical system | No recorded request at recent years, but common in modern systems. |
| EmfSensor | Sensor less |  |

Tuning support

| Features |  | remarks |
| --- | --- | --- |
| Sine generator for current/position loop identification |  |  |
| Fast “noise” generator for position identification |  |  |
| Step response test for current loop |  |  |
| Special internal procedure for current loop start-step |  |  |
| Internal FFT calculation for single testing frequency |  | To be done on PC. |

Simulation

| Features |  | remarks |
| --- | --- | --- |
| Plant simulation | Partially implemented as simulation sensor | Interface: to implement simulation of plant and mapping to the configured sensors. |

Other features

| Features | status | remarks |
| --- | --- | --- |
| Auto phasing | Not implemented in the latest versions, and shall not be supported | No recorded use in recent years |
| HOT CPU | Not implemented initially | Protocol shall be defined and communication lines reserved. For 2-axis systems with HOT CPU, one HOT CPU is assumed to handle both the axes. No mixed CLA-HOT options are planned |
| User program VM | Implementation exists | Must be integrated with resources of DSP ( flash for the user program co-existed with parameters sector, to be separated) |
| PC interface | Exists Delphi-written IDE. (sources remained in DftTuner) | Should be re-integrated with communication of DftTuner etc. In the nearest future to be re-written for modern GUI. |
| Interpreters | Mnemonic (XX[] ) and system function interpreter exist | Using this interpreter very slow updates Tuner GUI |
| MCAN | Basic MCAN HW drivers exist |  |
| Can Open | supported |  |
| CAN FD (1301 standard?) | Not implemented |  |
| 2-axes support | Control: For CPU+CLA supported For CPU+HotDsp yet not supported Ethercat: yet only single axis supported | Need little fixes for latest features (sensorless commutation) |